Z-80 Macro Library Documentation

--------------------------------

I.

The purpose of this library is to enable the assembly of the Z-80

instruction set on a CP/M sytem using the CP/M MAC macro assembler.

This library is invoked with the pseudo-op:

#"  MACLIB  Z80  "

II.

The following symbols and notations are used in the individual macro

descriptions;

r - Any of the 8 bit registers: A, B, C, D, E, H, L, or M

rr - Any of the 16 bit register pairs: BC, DE, HL, or SP

nn - 8 bit immediate data (0 through 255)

d  - 8 bit signed displacment (-128 through +127)

nnnn - 16 bit address or immediate data (0 through 65535)

b  - bit number (0-7, 7 is most significant, 0 is least)

addr - 16 bit address within PC+127 through PC-128

m(zzz) - Memory at address "zzz"

III.

MACLIB ver.		  Zilog ver		   TDL ver

--------------		-------------		-------------

LDX	r,d		LD	r,(IX+d)	MOV	r,d(IX)

Load register from indexed memory (with IX)

LDY	r,d		LD	r,(IY+d)	MOV	r,d(IY)

Load register from indexed memory (with IY)

STX	r,d		LD	(IX+d),r	MOV	d(IX),r

Store register to indexed memory (with IX)

STY	r,d		LD	(IY+d),r	MOV	d(IY),r

Store register to indexed memory (with IY)

MVIX	nn,d		LD	(IX+d),nn	MVI	d(IX)

Move immediate to indexed memory (with IX)

MVIY	nn,d		LD	(IY+d),nn	MVI	d(IY)

Move immediate to indexed memory (with IY)

LDAI			LD	A,I		LDAI

Move I to A

LDAR			LD	A,R		LDAR

Move R to A

STAI			LD	I,A		STAI

Move A to I

STAR			LD	R,A#		STAR

Move A to R

LXIX	nnnn		LD	IX,nnnn		LXI	IX,nnnn

Load IX immediate (16 bits)

LXIY	nnnn		LD	IY,nnnn		LXI	IY,nnnn

Load IY immediate (16 bits)

LBCD	nnnn		LD	BC,(nnnn)	LBCD	nnnn

Load BC direct (from memory at nnnn)

LDED	nnnn		LD	DE,(nnnn)	LDED	nnnn

Load DE direct

LSPD	nnnn		LD	SP,(nnnn)	LSPD	nnnn

Load SP direct

LIXD	nnnn		LD	IX,(nnnn)	LIXD	nnnn

Load IX direct

LIYD	nnnn		LD	IY,(nnnn)	LIYD	nnnn

Load IY direct

SBCD	nnnn		LD	(nnnn),BC	SBCD	nnnn

Store BC direct (to memory at nnnn)

SDED	nnnn		LD	(nnnn),DE	SDED	nnnn

Store DE direct

SSPD	nnnn		LD	(nnnn),SP	SSPD	nnnn

Store SP direct

SIXD	nnnn		LD	(nnnn),IX	SIXD	nnnn

Store IX direct

SIYD	nnnn		LD	(nnnn),IY	SIYD	nnnn

Store IY direct

SPIX			LD	SP,IX		SPIX

Copy IX to the SP

SPIY			LD	SP,IY		SPIY

Copy IY to the SP

PUSHIX			PUSH	IX		PUSH	IX

Push IX into the stack

PUSHIY			PUSH	IY		PUSH	IY

Push IY into the stack

POPIX			POP	IX		POP	IX

Pop IX from the stack

POPIY			POP	IY		POP	IY

Pop IY from the stack

EXAF			EX	AF,AF'		EXAF

Exchange AF and the alternate, AF'

EXX			EXX			EXX

Exchange BC DE HL with BC' DE' HL'

XTIX			EX	(SP),IX		XTIX

Exchange IX with the top of the stack

XTIY			EX	(SP),IY		XTIY

Exchange IY with the top of the stack

LDI			LDI			LDI

Move m(HL) to m(DE), increment DE and HL, decrement BC

LDIR			LDIR			LDIR

Repeat LDI until BC = 0

LDD			LDD			LDD

Move m(HL) to m(DE), decrement HL, DE, and BC

LDDR			LDDR			LDDR

Repeat LDD until BC = 0

CCI			CPI			CCI

Compare A with m(HL), increment HL, decrement BC

CCIR			CPIR			CCIR

Repeat CCI until BC = 0  or  A = m(HL)

CCD			CPD			CCD

Compare A with m(HL), decrement HL and BC

CCDR			CPDR			CCDR

Repeat CCD until BC = 0 or A = m(HL)

ADDX	d		ADD	(IX+d)		ADD	d(IX)

Indexed add to A

ADDY	d		ADD	(IY+d)		ADD	d(IY)

Indexed add to A

ADCX	d		ADC	(IX+d)		ADC	d(IX)

Indexed add with carry

ADCY	d		ADC	(IY+d)		ADC	d(IY)

Indexed add with carry

SUBX	d		SUB	(IX+d)		SUB	d(IX)

Indexed subtract

SUBY	d		SUB	(IY+d)		SUB	d(IY)

Indexed Subtract

SBCX	d		SBC	(IX+d)		SBB	d(IX)

Indexed subtract with "borrow"

SBCY	d		SBC	(IY+d)		SBB	d(IY)

Indexed subtract with borrow

ANDX	d		AND	(IX+d)		ANA	d(IX)

Indexed logical and

ANDY	d		AND	(IY+d)		ANA	d(IY)

Indexed logical and

XORX	d		XOR	(IX+d)		XRA	d(IX)

Indexed logical exclusive or

XORY	d		XOR	(IY+d)		XRA	d(IY)

Indexed logical exclusive or

ORX	d		OR	(IX+d)		ORA	d(IX)

Indexed logical or

ORY	d		OR	(IY+d)		ORA	d(IY)

Indexed logical exclusive or

CMPX	d		CP	(IX+d)		CMP	d(IX)

Indexed compare

CMPY	d		CP	(IY+d)		CMP	d(IY)

Index compare

INRX	d		INC	(IX+d)		INR	d(IX)

Increment memory at m(IX+d)

INRY	d		INC	(IY+d)		INR	d(IY)

Increment memory at m(IY+d)

DCRX	d		INC	(IX+d)		INR	d(IX)

Decrement memory at m(IX+d)

DCRY	d		DEC	(IY+d)		DCR	d(IY)

Decrement memory at m(IX+d)

NEG			NEG			NEG

Negate A (two's complement)

IM0			IM0			IM0

Set interrupt mode 0

IM1			IM1			IM1

Set interrupt mode 1

IM2			IM2			IM2

Set interrupt mode 2

DADC	rr		ADC	HL,rr		DADC	rr

Add with carry rr to HL

DSBC	rr		SBC	HL,rr		DSBC#	rr

Subtract with "borrow" rr from HL

DADX	rr		ADD#	IX,rr		DADX	rr

Add rr to IX (rr may be BC, DE, SP, IX)

DADY	rr		ADD	IY,rr		DADY#	rr

Add rr to IY (rr may be BC, DE, SP, IY)

INXIX			INC	IX		INX	IX

Increment IX

INXIY			INC	IY		INX	IY

Increment IY

DCXIX			DEC	IX		DCX	IX

Decrement IX

DCXIY			DEC	IY		DCX	IY

Decrement IY

BIT	b,r		BIT	b,r		BIT	b,r

Test bit b in register r

SETB	b,r		SET	b,r		SET	b,r

Set bit b in register r

RES	b,r		RES	b,r		RES	b,r

Reset bit b in register r

BITX	b,d		BIT	b,(IX+d)	BIT	b,d(IX)

Test bit b in memory at m(IX+d)

BITY	b,d		BIT	b,(IY+d)	BIT	b,d(IY)

Test bit b in memory at m(IY+d)

SETX	b,d		SET	b,(IX+d)	SET#	b,d(IX)

Set bit b in memory at m(IX+d)

SETY	b,d		SET	b,(IY+d)	SET	b,d(IY)

Set bit b in memory at m(IY+d)

RESX	b,d		RES	b,(IX+d)	RES	b,d(IX)

Reset bit b in memory at m(IX+d)

RESY	b,d		RES	b,(IY+d)	RES	b,d(IY)

Reset bit b in memory at m(IY+d)

JR	addr		JR	addr-$		JMPR	addr

Jump relative unconditional

JRC	addr		JR	C,addr-$	JRC	addr

Jump relative if Carry indicator true

JRNC	addr		JR	NC,addr-$	JRNC	addr

Jump relative if carry indicator false

JRZ	addr		JR	Z,addr-$	JRC	addr

Jump relative if Zero indicator true

JRNZ	addr		JR	NZ,addr-$	JRNZ	addr

Jump relative if Zero indicator false

DJNZ	addr		DJNZ	addr-$		DJNZ	addr

Decrement B, jump relative if non-zero

PCIX			JMP	(IX)		PCIX

Jump to address in IX ie, Load PC from IX

PCIY			JMP	(IY)		PCIY

Jump to address in IY

RETI			RETI			RETI

Return from interrupt

RETN			RETN			RETN

Return from non-maskable interrupt

INP	r		IN	r,(C)		INP	r

Input from port C to register r

OUTP	r		OUT	(C),r		OUTP	r

Output from register r to port (C)

INI			INI			INI

Input from port (C) to m(HL), increment HL, decrement b

INIR			INIR			INIR

Input from port (C) to m(HL), increment HL, decrement B, repeat if B <> 0

OUTI			OTI			OUTI

Output from m(HL) to port (C), increment HL, decrement B

OUTIR			OTIR			OUTIR

Repeat OUTI until B = 0

IND			IND			IND

Input from port (C) to m(HL), decrement HL & B

INDR			INDR			INDR

Repeat IND until B = 0

OUTD			OTD			OUTD

Output from m(HL) to port (C), decrement HL & B

OUTDR			OTDR			OUTDR

Repeat OUTD until B = 0

RLCR	r		RLC	r		RLCR	r

Rotate left circular register

RLCX	d		RLC	(IX+d)		RLCR	d(IX)

Rotate left circular indexed memory

RLCY	d		RLC	(IY+d)		RLCR	d(IY)

Rotate left circular indexed memory

RALR	r		RL	r		RALR	r

Rotate left arithmetic register

RALX	d		RL	(IX+d)		RALR	d(IX)

Rotate left arithmetic indexed memory

RALY	d		RL	(IY+d)		RALR	d(IY)

Rotate left arithmetic indexed memory

RRCR	r		RRC	r		RRCR	r

Rotate right circular register

RRCX	d		RRC	(IX+d)		RRCR	d(IX)

Rotate right circular indexed

RRCY#	d		RRC	(IY+d)		RRCR	d(IY)

Rotate right circular indexed

RARR	r		RR	r		RARR	r

Rotate right arithmetic register

RARX	d		RR	(IX+d)		RARR	d(IX)

Rotate right arithmetic indexed memory

RARY#	d		RR	(IY+d)		RARR	d(IY)

Rotate right arithmetic indexed memory

SLAR	r		SLA	r		SLAR	r

Shift left register

SLAX	d		SLA	(IX+d)		SLAR	d(IX)

Shift left indexed memory

SLAY	d		SLA	(IY+d)		SLAR	d(IY)

Shift left indexed memory

SRAR	r		SRA	r		SRAR	r

Shift right arithmetic register

SRAX	d		SRA	(IX+d)		SRAR	d(IX)

Shift right arithmetic indexed memory

SRAY	d		SRA	(IY+d)		SRAR	d(IY)

Shift right arithmetic indexed memory

SRLR	r		SRL	r		SRLR	r

Shift right logical register

SRLX	d		SRL	(IX+d)		SRLR	d(IX)

Shift right logical indexed memory

SRLY	d		SRL	(IY+d)		SRLR	d(IY)

Shift right logical indexed memory

RLD			RLD			RLD

Rotate left digit

RRD			RRD			RRD

Rotate right digit

############################################################################################################HK	?D

DB	0FDH, 0CBH, ?D, 16H

ENDM

RRCR	MACRO	?R

DB	0CBH, 08H + ?R

ENDM

RRCX	MACRO	?D

@CHK	?D

DB	0DDH, 0CBH, ?D, 0EH

ENDM

RRCY	MACRO	?D

@CHK	?D

DB	0FDH, 0CBH, ?D, 0EH

ENDM

RARR	MACRO	?R

DB	0CBH, 18H + ?R

ENDM

RARX	MACRO	?D